| PROJECT ID: 24HPAI03 | SDG 03 — GOOD HEALTH & WELL-BEING | ATRIA INSTITUTE OF TECHNOLOGY |
| --- | --- | --- |

WHEEZING PRE-DETECTION & ALERT SYSTEM AI-Powered Respiratory Monitoring ¦ Early Warning ¦ Preventive Healthcare

Design Thinking Methodology

| 01 Empathize Field visits with Mrs. Nandini, Mrs. Kumari & Dr. Sheela confirmed unmet need for early warning. | 02 Define No affordable system predicts wheezing before onset — causing panic and emergency hospitalization. | 03 Ideate Acoustic sensor + ML pre-detection + multi-level alert system for asthma & COPD patients. | 04 Prototype Wearable mic + ESP32 MCU + CNN/LSTM model + mobile app alert pipeline with cloud logging. | 05 Test & Validate Validated with pulmonologist feedback; model retrained iteratively on real breathing datasets. |
| --- | --- | --- | --- | --- |

System Overview

| Wheezing Pre-Detection & Alert System. ▼ Step 1 Sensor Data Collection Breathing sounds & environmental data. ▼ Step 2 Signal Processing Filter noise · Extract audio features. ▼ Step 3 AI / ML Analysis Pattern recognition · Predict attack risk. ▼ Risk Detected? YES ▼ ¦ NO → Continue Monitoring. ▼ Step 4 Trigger Alert Buzzer · LED · Mobile notification. ▼ Step 5 Caregiver / Doctor Notified SMS / App alert to guardian or hospital. ▼ Step 6 Timely Intervention Inhaler use · Medical aid · Avoid ER visit. ▼ OUTCOME: Attack Prevented Improved quality of life · Reduced healthcare cost. | Technical Stack MICROCONTROLLER ESP32 MCU; ML MODELS CNN + LSTM. LANGUAGE Python 3.x; DL FRAMEWORK TensorFlow / Keras. SIGNAL PROCESSING LibROSA / MFCC; CLOUD BACKEND Firebase / IoT Hub. MOBILE ALERT Flutter App + SMS; AUDIO FEATURES MFCC · Mel Spectrogram. Hardware Components ID; Component; Description. H01; Electret Microphone / MAX9814; Wearable mic for continuous breathing sound capture. H02; ESP32 Dev Board; Main MCU — Wi-Fi + Bluetooth + ADC + processing. H03; Active Buzzer Module; Audible alert triggered on risk detection event. H04; RGB LED Indicator; Visual status — Green: safe · Red: risk detected. H05; DHT22 Sensor; Ambient temperature & humidity monitoring. H06; Li-Po Battery + TP4056; Rechargeable 3.7V portable power supply. H07; 3D Printed Enclosure; Wearable chest-mount housing for patient comfort. |
| --- | --- |

Societal Impact

| Healthcare Impact Reduces emergency hospitalizations and ER visits through proactive early warning before a wheezing attack begins. | Economic Impact Lowers treatment and hospitalization costs for patients and the healthcare system through affordable device design. | Social Impact Empowers asthma and COPD patients in rural and urban communities with safer, more dignified daily living. |
| --- | --- | --- |

| Atria Institute of Technology (Autonomous), Bengaluru | Team: Suraj T · Ramya Charitha · Tanushri MP · Kishor S · Manohar · Mahalakshmi |
| --- | --- |